Ground-coupled heat pump systems are environmentally friendly when, compared with existing heating and cooling equipment because it is emitting relatively low amount of greenhouse gases such as, carbon dioxide. Geothermal heat pump system, is also being recognized as renewable energy sources that can significantly decreased energy, of building sector. Lund and Boyd revealed that; between 82 countries that applies the direct use of the geothermal energy, 43 have distributed geothermal heat pump systems. Based on the 2014 data , the global geothermal heat pump installation capacity is approximately 49.9 GWth, which is 50.8% increase on comparison to year 2010 (33.1 GWth).

Among the various types of geothermal heat pump systems, borehole ground heat exchanger is most commonly used . When the heating and cooling energy load of the building studied achieves balance the geothermal system has significant effect. Conversely in cooling-dominated building, the system’s performance is decreases because vertical ground heat exchanger (VGHE) circulating water temperature continuously risen. Meanwhile, in heating-dominated buildings the opposite phenomenon of cooling occurs; which is the reason, system’s heating performance decreases. A hybrid ground-coupled heat pump system, is being recognized as an alternative on this.

Hybrid geothermal heat pump systems are considered to reduce initial construction costs, while simultaneously maintaining stable system performance. This system uses auxiliary heat sources (boiler and solar power) or auxiliary heat sink (surface water) in same manner to a primary heat source. Ground temperature (or ground circulating water temperature) can be balanced, by connecting these types of the auxiliary equipment. When constructing geothermal system, it is normally matched to whichever side, of heating and cooling load, that was larger, and a ground heat exchanger is constructed. Thus, one side having the higher load, than necessary. Cooling-dominated buildings install ground heat exchangers to match heating load, and can process the insufficient cooling load by surface water.

This present thesis analyzing the effect by surface water heat exchanger (SWHE) design parameters on length of heat exchanger pipes. This study applied effectiveness-NTU method, one of the typical heat exchange design techniques, and used existing heat transfer correlation. The present study was limited to shallow ponds, and considered pond temperature and pipe specifications as design parameters. Next, this study analyze the application effect, of SWHE and VGHE, after assuming that they are used together.

The conclusions are as follows.

(1) More heat exchanger pipes are needed, the closer the SWHE outlet temperature approaches to pond water temperature.

(2) Length somewhat decreases the larger the outside diameter is, of the heat exchanger pipe. However, there was virtually no outside diameter effect, when outlet temperature is higher than approximately 29℃.

(3) Pipe length decreases, when SDR value is large, because thermal resistance, which effects to heat transfer, decreased.

(4) In a state, where cooling design entering source temperature (EST) is fixed, VGHE length can be reduced if it will be used together with SWHE.